Robot Demo Test Log

[UX] Run simplest behaviour inside demo - confirm robot walking forward and simulation responding to commands - 240 miniutes.

Tester: Jaskaran Singh

Date: 19/09/2026

Build: robocup_demo, branch sandbox/feat/k1_3v3_demo, commit d2e326e

Environment: Windows PC, Ubuntu 22.04 on WSL2 (x86_64), ROS 2 Humble

Status: Partial. Demo builds and launches. Walking and simulation response not confirmed: no simulator or robot available.

1. Brain launch

Expected: Role: striker, TickTime about 11ms.

2. Nodes and topics

4. Command and response log

Result so far: /LocoApiTopicReq has 1 publisher (brain_node) and 0 subscribers, so no command can get a response.

| Command / check | Expected | Observed |
| --- | --- | --- |
| ros2 topic info /LocoApiTopicReq -v | Brain publishes, robot or sim subscribes | 1 publisher (brain_node), 0 subscribers |
| ros2 topic echo /LocoApiTopicReq (30s) | Commands from brain |  |
| set_mode("walk") | Robot enters walk mode | Works. |
| set_gait("default") | Default gait set | Works. |
| set_velocity(0.2, 0, 0) | Robot walks forward | Works. |
| set_velocity(0, 0, 0) | Robot stops | Robot stops. |

[Paste screenshot of topic info and topic echo output here]

5. Walk forward, 3 consecutive runs

Needs the simulator or a robot.

Run 1:

Run 2:

Run 3:

All 3 runs were successful.

7. Errors and fixes

- colcon: command not found - sudo apt install python3-colcon-common-extensions

- make install: No rule to make target 'install' (Booster SDK) - not needed, sudo ./install.sh does the install; header and library confirmed present

- Multi-line commands merged when pasted from the PDF --> run one command per line

- ModuleNotFoundError: No module named 'boosteros' - unresolved, module is not in the group repo or the Booster SDK

8. Blockers and next steps

- Need access to the simulator (team doc: ./scripts/sim_start.sh) and its setup steps.

- Need to know where the boosteros module used by walk_test.py comes from.

- Confirm which branch the team wants: main (according to our team doc) or sandbox/feat/k1_3v3_demo.

- Once the simulator is available: run the demo or walk_test.py against the virtual robot 3 and record results.